CCNA: Introduction to Network

Module 4 – 4.3: Coper Cabling (Cáp đồng)

A. Characteristics of Copper Cabling

- Copper cabling is the most common type of cabling used in networks today.

- Networks use copper media because it is inexpensive, easy to install, and has low resistance to electrical current. However, copper media is limited by distance and signal interference (nhiễu tín hiệu).

- Data is transmitted on copper cables as electrical pulses (xung điện).

- the farther the signal travels, the more it deteriorates (suy giàm). This is referred to as signal attenuation (suy giảm tín hiệu).

=> All copper media must follow strict distance limitations as specified by the guiding standards.

- Electrical signals are vulnerable to interference:

- EMI (Electromagnetic interference) /RFI (radio frequency interference): Distortion from radio waves and electromagnetic devices (e.g., lights, motors).

- Crosstalk: Interference between signals in adjacent wires.

- To counter the negative effects of EMI and RFI, some types of copper cables are wrapped in metallic shielding (lớp chắn kim loại) and require proper grounding connections.

- To counter the negative effects of crosstalk (nhiễu xuyên âm), some types of copper cables have opposing circuit wire pairs twisted together, which effectively cancels the crosstalk.

- The susceptibility of copper cables to electronic noise can also be limited using these recommendations:

- Selecting the cable type or category most suited to a given networking environment

- Designing a cable infrastructure to avoid known and potential sources of interference in the building structure

- Using cabling techniques that include the proper handling and termination of the cables

B. Types of Copper Cabling

B. Unshielded Twisted-Pair (UTP) Cable

- UTP is common: Unshielded twisted-pair cable is the most popular networking cable.

- RJ-45 connectors: UTP cables use RJ-45 connectors to connect devices.

- LAN use: UTP connects computers to network devices like switches and routers in local area networks (LANs).

- Four twisted pairs: UTP cable contains four pairs of twisted wires inside a protective sheath.

- Twisting reduces interference: The twisting of the wires helps prevent signal interference.

The numbers in the figure identify some key characteristics of unshielded twisted-pair cable:

- The outer jacket protects the copper wires from physical damage.

- Twisted-pairs protect the signal from interference.

- Color-coded plastic insulation electrically isolates wires from each other and identifies each pair.

C. Shielded Twisted-Pair (STP) Cable

- STP offers better protection: Shielded twisted-pair cable (STP) provides better protection against EMI and RFI than UTP.

- More expensive and harder to install: STP is more costly and complex to install than UTP.

- Uses RJ-45 connectors: Like UTP, STP also uses RJ-45 connectors (though shielded versions exist).

- Shielding and twisting: STP uses both shielding (foil or braid) and wire twisting to minimize interference.

- Proper grounding is crucial: Shielding needs proper grounding to work effectively; otherwise, it can worsen interference.

- Four shielded pairs: STP typically contains four wire pairs, each individually shielded and then wrapped together.

The numbers in the figure identify some key features of shielded twisted-pair cable:

- Outer jacket

- Braided or foil shield

- Foil shields

- Twisted pairs

D. Coaxial Cable

- Coax Structure: Coaxial cable has two conductors sharing the same axis: a central copper conductor and a surrounding metallic shield (braid or foil), separated by insulation.

- Shielding: The shield protects the signal from electromagnetic interference.

- Connectors: Coax uses different connectors like BNC, N type, and F type.

- Modern Use Cases: While less common for Ethernet, coax is still used for:

- Wireless: Connecting antennas to wireless devices.

- Cable Internet: Delivering internet service to homes (though fiber is replacing some segments).

The numbers in the figure identify some key features of coaxial cable:

- Outer jacket

- Braided copper shielding

- Plastic insulation

- Copper conductor